# Notice of Determination

May 5th, 2025 – Transport Canada has determined that the proposed tank removal by AVJET is not likely to cause significant adverse environmental effects.

This determination was based on a consideration of the following factors:

- impacts on rights of Indigenous peoples;

- Indigenous knowledge;

- community knowledge;

- comments received from the public; and

- technically and economically feasible mitigation measures.

Mitigation measures taken into account for this determination are:

- Worksite (General)

- It is the contractor's responsibility to identify and locate underground infrastructures before proceeding with any excavation work.

- Keep site and adjacent areas clean. Clean the site as work progresses to prevent the accumulation of dust, dirt, debris, materials and other waste.

- Ensure at all times that no waste is left behind and that the site is returned to an environmentally friendly condition at the end of the work.

- Accidents and malfunctions

- Set up a spill prevention and response plan, clearly identifying the people and organizations to contact and the procedure to follow in the event of an environmental emergency.

- A spill kit must be on site at all times.

- Any spill must be quickly brought under control, and contaminated material must be managed in accordance with current standards. Recover and dispose of any contamination, whether spilled product, soil or water, in accordance with applicable regulations. Notify the airport manager immediately. The characterization and remediation work, and the remediation report, must be carried out by an environmental consulting firm.

- Any spill with environmental consequences must be reported to the emergency services of Environment and Climate Change Canada (ECCC) (1-866-283-2333) and to the emergency service of the Ministère de l'Environnement et de la Lutte contre les changements climatiques, de la Faune et des Parc (MELCCFP) (1-866-694-5454).

- Use of machinery

- Ensure that equipment and machinery are in good working order and free of hydrocarbon leaks.

- Maintain, clean and refuel equipment and vehicles in designated areas on a watertight surface, at least 30 m from watercourses and 15 m from drainage ditches.

- Storage tank decommissioning and removal

- Tanks to be removed are registered with Environment and Climate Change Canada (ECCC) and are subject to the Storage Tank Systems for Petroleum Products and Allied Petroleum Products Regulations (STSOPP). The permanent decommissioning and removal of a storage tank system subject to RSTOPP must comply with sections 44 and 45 of these regulations. Tank decommissioning and removal must be carried out by a person certified by the RBQ.

- All operations must be carried out in such a way as to prevent any harmful effects on the environment.

- The owner must record information on decommissioning and removal in a register and notify ECCC of the removal and decommissioning of these tanks.

- Excavation - management of spoil or soil

- If work is to be carried out near an observation well, the contractor must take the necessary measures to preserve the facility.

- It is the developer's responsibility to ensure that the infrastructure is located prior to the work, at his own expense.

- Soil characterization (left in place) must be carried out by a representative of the environmental consultant.

- In the event of excavation of soils showing signs of contamination, they must also be stored between two watertight membranes, analyzed by an accredited laboratory and managed according to their levels of contamination, in compliance with current regulations. These soils or materials must not be reused on site, and must be managed off-site, in accordance with the Grille de gestion des sols excavés of the Guide d'intervention - Protection des sols et réhabilitation des terrains contaminés of the Ministère de l'Environnement, de la Lutte contre les changements climatiques, de la Faune et des Parcs (MELCCFP Guide).

- The proponent must ensure that off-site transportation of excavated soil is carried out in compliance with the Règlement concernant la traçabilité des sols contaminés excavés. This applies to all contaminated soils excavated from human activity in the A-B range, regardless of volume.

- Cover dump trucks when transporting fine materials.

- All imported backfill material must be clean, i.e. below criterion A of the MELCCFP's Guide d'intervention, Protection des sols et réhabilitation des terrains contaminés (“The Guide”).

- Residual materials management

- Adopt management practices for construction, renovation and demolition residues, in particular the recovery and reclamation of concrete residues (aggregate residues).

- Ensure at all times that no waste or excavated material is left behind and that the site is returned to an environmentally friendly condition at the end of the work.

Transport Canada is satisfied that that the carrying out of the project is not likely to cause significant adverse environmental effects. Therefore, Transport Canada exercise any power, or perform any duty or function, to enable the project to be carried out.